# Automated Sorting Station – PLC & HMI Control

Abdelrahman Elfeky

# Project Overview

This project implements an automated conveyor-based sorting station controlled by a Siemens S7-1500 PLC and operated through a WinCC Unified HMI. The system is designed to simulate an industrial material handling station, focusing on correct control logic, safe operation, alarm handling, and professional HMI design.

Factory I/O is used to simulate the physical process, including conveyors, sensors, and operator control devices. The project emphasizes realistic industrial practices rather than academic or purely theoretical control logic.

# System Architecture

The system consists of the following main components:

- PLC: Siemens S7-1500, responsible for all control logic, state handling, and safety interlocks

- HMI: WinCC Unified, providing operator control, status visualization, and alarm handling

- Simulation Environment: Factory I/O, simulating field devices and material flow

### Field Devices (Simulated):

  - Conveyor motor

  - Presence / detection sensors

  - Emergency stop pushbutton

  - Operator control buttons (Start, Stop, Reset, Manual controls)

The PLC communicates with the HMI for operator interaction and with Factory I/O for real-time process simulation.

# Control Philosophy

## Operating Modes

The system operates in two distinct modes:

### Automatic Mode:

      - The conveyor operates based on PLC-controlled sequencing logic

      - Box detection and process flow are handled automatically

### Manual Mode:

      - Intended for commissioning, maintenance, and troubleshooting

      - The operator directly controls machine actions through the HMI

      - All safety interlocks remain active

Mode selection is handled explicitly, and only one mode can be active at a time.

## Manual Control Concept

In manual mode, the operator can control the machine using dedicated HMI buttons. This allows controlled movement of the conveyor and associated actuators without relying on automatic sequencing.

Manual control is only permitted when: - The system is not in automatic mode - No safety or process faults are active - Emergency stop is released

This design reflects common industrial practice, where manual mode is used exclusively for non- production activities.

## Start, Stop, and Reset Behavior

    - Start: Enables machine operation according to the selected mode

    - Stop: Brings the system to a controlled stop

    - Reset: Clears latched faults and alarms after the fault condition has been resolved The system does not allow restarting while safety conditions are violated.

# Safety Concept

Safety has priority over all control logic.

## Emergency Stop

    - Emergency stop activation immediately stops all conveyor motion

    - A latched safety alarm is triggered

    - Operation can only resume after:

      - The emergency stop is released

      - The operator performs a reset

This ensures controlled recovery and prevents unexpected restarts.

# Alarm Management

The system implements alarms to clearly inform the operator of abnormal or unsafe conditions.

## Alarm Philosophy

    - Alarms are latched where appropriate

    - Operator acknowledgment is required

    - Alarm texts are written in clear, operator-friendly language

## Implemented Alarms

Alarm	Description

Emergency Stop Activated

Stopblade Jam Detected

Triggered when the emergency stop is pressed

Stopblade stuck in ON mode.

Sorter error	Sorter condition violated

# HMI Design

The HMI is designed to be simple, clear, and operator-focused.

## Overview Screen

The overview screen provides: - Machine status indication - Mode indication (Manual / Automatic) - Start, Stop, and Reset controls - Active alarm indication

## Manual Mode screen

This screen allows the operator to control the whole system manually. It’s provided with buttons for easy control and LEDs to make it simple for the operator to know the status of the actuators.

# Testing and Validation

The system was tested using Factory I/O by simulating: - Normal automatic operation - Manual operation via HMI controls - Emergency stop activation and recovery - Process fault scenarios such as conveyor blockage

All safety responses, alarms, and control behaviors are operated as intended.

# 9. Conclusion

This project demonstrates the implementation of an industrial-style PLC control system with a strong focus on safety, structured control logic, alarm management, and professional HMI design. The solution reflects real-world automation practices and is suitable as a portfolio project for entry-level automation and control engineering roles.